Multiply

LDI R16, 0xA1

LDI R17, 0xA

MUL R16, R17

Block transfer

; Block Transfer Program for ATmega32

; Copies 10 bytes from source (starting at 0x60) to destination (starting at 0x70)

.def temp = r16        ; temporary register

.def counter = r17     ; loop counter

ldi counter, 10        ; number of bytes to transfer

ldi r30, 0x60          ; Z-register low (source pointer)

ldi r31, 0x00          ; Z-register high

ldi r26, 0x70          ; X-register low (destination pointer)

ldi r27, 0x00          ; X-register high

loop:

ld  temp, Z+           ; load from source, post-increment Z

st  X+, temp           ; store to destination, post-increment X

dec counter            ; decrement counter

brne loop              ; repeat until counter = 0

done:

rjmp done              ; infinite loop (stop)

;

; fibonacci_series.asm

;

; Created: 11/30/2025 8:48:27 PM

; Author :

;

start:

ldi r16, 0x00 ;load r16 with first fibo number 0

ldi r17, 0x01 ; load r17 with 2nd fibo number 1

ldi r18, 0x0A ;load r18 with the number of numbers to generate

ldi xh, 0x00 ; load the high byte of the addr (0x0060)

ldi xl, 0x60 ; load the low byte of the addr (0x0060)

st x+, r16 ;store the first numbe 0 at the memory location 0x0060

st x+, r17 ;store the second fibonacci numbe 1 at teh next memory location 0x0061

loop:

mov r19, r16 ; copy the value of previous fibo number to r19

add r19, r17 ; add r19 and r17 to get the next fibo numbe

st x+, r19 ;store the new fibo number in the new memory location

mov r16, r17 ;Update r 16 to the next numbe in the seq (prev become current)

mov r17, r19 ; update r17 to the newly calculated number

dec r18 ; decrement counter (number of fibo numbers to generate)

brne loop ; if r18 is not zero, repeat the loop to generate the next numbe

maxima minima

; program to find maximum numbers in an array of numbers

; Program to find maximum number in an array of 5 numbers

start:

ldi r16, 0x05        ; number of elements

ldi ZH, 0x00         ; high byte of array address

ldi ZL, 0x60         ; low byte of array address

ld r18, Z+           ; load first element

mov r19, r18         ; initialize max = first element

dec r16              ; now 4 elements left

loop:

ld r18, Z+           ; load next element

cp r18, r19          ; compare with current max

brlo skip            ; if r18 < r19, skip

mov r19, r18         ; else update max

skip:

dec r16              ; decrement counter

brne loop            ; repeat until all elements checked

done:

rjmp done            ; infinite loop (result in r19)

asending desinding

; Program to sort an array using a simple sorting algorithm (Bubble Sort)

start:

LDI XH, 0x00        ; Load the high byte of the address of the array (0x0000)

LDI XL, 0x76        ; Load the low byte of the address of the array (0x0070)

LDI R16, 0x05       ; Load R16 with the number of elements to sort (5 elements)

sort_outer_loop:

LDI YH, 0x00        ; Load the high byte of the address for the inner loop (0x0000)

LDI YL, 0x60        ; Load the low byte of the starting address for sorting (0x0060)

LDI R17, 0x04       ; Load R17 with number of comparisons (elements - 1)

inner_loop:

LD R18, Y+          ; Load the first element from the array into R18 and increment Y

LD R19, Y           ; Load the second element from the array into R19 (Y not incremented)

; Compare the two elements

CP R18, R19         ; Compare the first and second elements

BRLO skip_swap      ; If R18 < R19 (first is smaller), skip the swap

; Swap values

MOV R20, R18        ; Move the first element (R18) to R20 (temporary storage)

MOV R18, R19        ; Move the second element (R19) into the first position (R18)

MOV R19, R20        ; Move the value from R20 back to the second position (R19)

ST Y, R19           ; Store the new value of R19 (original second element) back into the array

SBIW Y, 1           ; Decrement Y to point back to the first element

ST Y, R18           ; Store the new value of R18 (original first element) back into the array

; Prepare for next comparison

ADIW Y, 1           ; Increment Y to point to the next pair of elements

skip_swap:

DEC R17             ; Decrement the comparison counter

BRNE inner_loop     ; If more comparisons, repeat the inner loop

DEC R16             ; Decrement the outer loop counter

BRNE sort_outer_loop ; If more elements to sort, repeat the outer loop

RJMP start              ; Jump back to the start of the program (infinite loop)